Land Valuation Using the Market Method

Technical Assessment Response

Implementation Plan, System Design & Execution Model

Prepared by: Mohamed Raafat

April 202

# 1. Implementation Plan

The solution is divided into two stages. All weights, scoring brackets, and thresholds are externalized in a JSON configuration file (valuation_config.json), allowing adjustments without code changes.

## Stage 1: Benchmark Selection

This stage filters the transaction dataset and selects the 5 most similar comparable lands to the target property.

| Step | Action | Output |
| --- | --- | --- |
| 1a | Filter by target city only | City-scoped candidates |
| 1b | Basic filters: asset type, nulls, valid price/area, zoning, road width | Filtered dataset |
| 1c | Remove price outliers (P5/P95) within the city | Clean candidate pool |
| 2 | Compute derived metrics: price/sqm, area categories (P33/P66) | Enriched candidates |
| 3 | Compute weighted similarity score (non-price attributes) for each candidate vs target | Ranked candidates |
| 4 | Validate top 5: check for duplicates, area/price proximity | Validated benchmarks |
| 5 | Export benchmark set with per-feature similarity scores | Stage 1 output CSV |

## Stage 2: Market Valuation

This stage scores both the target and each benchmark across 10 weighted factors, then uses the point-value relationship to derive the target’s estimated price.

| Step | Action | Output |
| --- | --- | --- |
| 1 | Score target land across 10 factors (weight × multiplier) | Target total points |
| 2 | Score each benchmark across same 10 factors | Benchmark total points |
| 3 | Compute point value per benchmark: price_per_sqm / total_points | Point values |
| 4 | Weighted average point value (using Stage 1 similarity as weights) | Avg point value |
| 5 | Final price: avg_point_value × target_total_points | Estimated price/sqm |

## Configuration-Driven Design

All tunable parameters are stored in a single JSON configuration file, including Stage 1 similarity weights and top-K count, Stage 2 factor weights, outlier removal percentiles, area category thresholds, all scoring brackets (percentage difference, distance in km/meters, grade mappings), zoning compatibility matrix, and address/location fallback rules. This means the entire valuation behavior can be adjusted by editing the config file without touching any code.

# 2. System Flow: Input to Final Valuation

The end-to-end flow from user input to valuation result follows this sequence:

## Input

The user provides the target land’s attributes: city, district, coordinates, area, zoning, road width, FAR/CSR, frontage/depth, distances to services/roads/commercial/admin centers, utility availability, and commercial/admin grades. No price is provided — that is what the system estimates.

## Processing Pipeline

Step 1 — Data Preparation: Load the transaction dataset, filter to the target’s city, apply basic quality filters (asset type, nulls, valid values), remove price outliers using P5/P95 thresholds computed within that city, and derive price/sqm and area categories.

Step 2 — Benchmark Selection: Compute a weighted similarity score between the target and every candidate using 9 non-price features (district, coordinates, area, zoning, road width, FAR/CSR, grades, utilities, recency). Features with missing data are skipped and their weight redistributed. Select the top 5 candidates. Validate for duplicates and excessive similarity.

Step 3 — Factor Scoring: Score both the target and each benchmark independently across 10 weighted factors. Three factors are relative (area, general location, FAR/CSR compare the comparable to the target), while seven are absolute (scored based on the property’s own attributes).

Step 4 — Pricing: Derive point value per benchmark (price/sqm ÷ total points), compute a weighted average using Stage 1 similarity scores, and multiply by the target’s total points to get the estimated price/sqm.

## Output

The system produces three output sets: benchmark details with similarity scores, factor-by-factor scoring for target and each benchmark, and the final estimated price per sqm with optional total land value.

# 3. Backend Requirements

## Data Storage

- Transaction database: all historical land transactions with full attribute set, updated periodically

- Benchmark cache: selected benchmarks per target valuation, with similarity scores and selection metadata

- Scoring results store: per-run audit trail with every factor score, weight, multiplier, and final valuation

- Configuration store: admin-editable weights, scoring brackets, and factor activation flags

## Computation Requirements

- Similarity scoring: O(N) per valuation where N = candidates in the target city after filtering

- Factor scoring: O(K × F) where K = 5 benchmarks and F = 10 factors — negligible compute

- Outlier detection: percentile computation on city-level data, efficient with pandas

- Haversine distance: computed per candidate for coordinate-based scoring

## Auto-Refresh Logic

The business case requires benchmarks to be re-selected monthly/quarterly or when new transactions exceed a threshold. This can be implemented as a scheduled job that re-runs the Stage 1 pipeline for active valuations and flags any significant changes in benchmark composition.

# 4. Integrations, APIs & Data Sources

## Data Sources

| Source | Purpose | Format |
| --- | --- | --- |
| Land transaction records | Primary dataset for benchmarking and pricing | CSV / Database table |
| Geocoding service (e.g., Google Maps API) | Convert addresses to coordinates when missing | REST API |
| Zoning registry | Validate zoning classifications and FAR/CSR values | Database / API |
| GIS / Spatial data | Distance calculations to services, roads, commercial areas | Geospatial DB |

## API Endpoints (Production)

| Endpoint | Method | Description |
| --- | --- | --- |
| /api/valuate | POST | Submit target land attributes, receive full valuation result |
| /api/benchmarks | GET | Retrieve current benchmarks for a given target |
| /api/config/weights | GET/PUT | Read or update factor weights and scoring brackets |
| /api/audit/{run_id} | GET | Retrieve full audit trail for a specific valuation run |

## Key Calculations

Similarity Score: Weighted sum of 9 feature-level scores (district match, coordinate proximity, area closeness, zoning compatibility, road width, FAR/CSR, grades, utilities, recency). Missing features are skipped and weights redistributed.

Factor Score: s_i = w_i × m_i, where w_i is the factor weight and m_i is a multiplier between 0 and 1 derived from business-case brackets.

Point Value: V_j = price_per_sqm_j / P_j, representing the EGP value of one scoring point for comparable j.

Target Price: p_target = V̄ × P_target, where V̄ is the weighted average point value across all benchmarks.

# 5. Data Model

The data model follows the four-table structure specified in the business case:

## A. Subject Record

| Field | Type | Description |
| --- | --- | --- |
| subject_id | String | Unique identifier for the target land |
| city, district | String | Administrative location |
| latitude, longitude | Float | Geographic coordinates |
| area_sqm | Float | Total area in square meters |
| zoning | String | Zoning classification (Residential, Commercial, Mixed Use, etc.) |
| road_width_m | Float | Adjacent road width in meters |
| far, csr | Float | Floor Area Ratio and Coverage Site Ratio |
| frontage_m, depth_m | Float | Plot dimensions |
| dist_to_commercial_m | Float | Distance to nearest commercial corridor |
| dist_to_admin_m | Float | Distance to nearest administrative center |
| dist_to_services_m | Float | Distance to nearest services |
| dist_to_main_road_m | Float | Distance to nearest main road |
| commercial_grade | String | Commercial location grade (A/B/C/D/NonCommercial) |
| admin_grade | String | Administrative location grade (A/B/C/D) |
| utilities_count | Integer | Number of available utilities (0–5) |

## B. Comparable Records

Same structure as the subject record, plus: comp_price_per_sqm (Float), transaction_date (Date), and similarity_score (Float, from Stage 1).

## C. Factors Configuration

| Field | Type | Description |
| --- | --- | --- |
| factor_key | String | Unique factor identifier (e.g., “address”, “area”) |
| weight | Float | Factor weight (all weights sum to 1.0) |
| scoring_method | String | Method type: pct_diff, distance_km, grade_map, count_based, etc. |
| params | JSON | Thresholds, bracket definitions, mappings, and tolerances |
| is_active | Boolean | Whether this factor is included in scoring |

## D. Scoring Results (Audit Trail)

| Field | Type | Description |
| --- | --- | --- |
| run_id | String | Unique identifier per valuation run |
| run_date | DateTime | Timestamp of the valuation |
| asset_id | String | Subject or comparable land ID |
| record_type | String | target / benchmark / result |
| factor_key | String | Which factor this score applies to |
| weight | Float | Weight used for this factor |
| multiplier | Float | Multiplier computed (0–1) |
| score_points | Float | weight × multiplier |
| total_points | Float | Sum of all factor scores for this property |
| point_value | Float | price_per_sqm / total_points (benchmarks only) |
| similarity_score | Float | Stage 1 similarity to target (benchmarks only) |
| raw_inputs_snapshot | JSON | Input values used for this scoring (for reproducibility) |

# 6. Expected Final Output for Business User

For each valuation run, the system delivers three output sets:

## Benchmark Output (Stage 1)

- 5 selected comparable lands with Land ID, city, district, area, price/sqm, zoning, and road width

- Similarity score and per-feature score breakdown (district, coordinates, area, zoning, road width, FAR/CSR, grades, utilities, recency)

- Area category classification and transaction date

## Comparable Scoring Output (Stage 2)

- For each benchmark: all 10 factor multipliers and weighted scores, total points, and point value

- For the target: all 10 factor multipliers and weighted scores, total points

- Eligibility status: all comparables passed city-level filtering in Stage 1

## Valuation Result

- Target total points

- Weighted average point value (using similarity scores as weights) and simple average for comparison

- Final estimated price per square meter

- Total estimated land value (price/sqm × area)

- Full audit trail: weights used, scoring logic applied, date of run, all stored inputs and outputs

## Example Output Summary

| Metric | Value |
| --- | --- |
| Target City | Cairo |
| Target District | Nasr City |
| Target Area | 400 sqm |
| Target Zoning | Residential |
| Target Total Points | 0.9400 |
| Weighted Avg Point Value | 39,173.87 EGP/point |
| Estimated Price/sqm | 36,823.44 EGP |
| Total Estimated Land Value | 14,729,375 EGP |
| Benchmarks Used | 5 (similarity range: 0.53 – 0.61) |

# 7. Assumptions, Dependencies & Risks

## Assumptions

- Transaction prices in the dataset reflect arm’s-length market transactions (not distressed or related-party sales)

- Zoning classifications are consistent across the dataset and accurately recorded

- Coordinates, when available, are sufficiently accurate for distance-based scoring

- The target land’s attributes are provided accurately by the user

- Synthetic data was used for this prototype; real data will follow the same schema

- All prices are in EGP and areas in square meters (no currency/unit conversion needed)

## Dependencies

- Sufficient transaction volume within the target city for meaningful comparison (minimum ~50 clean records)

- Regular data updates to keep benchmarks current (monthly or quarterly refresh cycle)

- Geocoding service availability for distance calculations when coordinates are missing

- Admin configuration of weights and scoring brackets aligned with current market conditions and policy

## Risks & Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Sparse data in certain cities/zones | High | Fall back to broader geographic scope; flag low-confidence valuations when candidate pool < 20 |
| Stale transaction data | Medium | Recency scoring penalizes old transactions; auto-refresh re-runs pipeline periodically |
| Missing attributes (coordinates, distances) | Medium | Weight redistribution skips missing features; fallback scoring (e.g., district-based for location) |
| Outlier transactions surviving P5/P95 filter | Low | Validation step checks for suspiciously identical benchmarks; business users can review/override |
| Weight misconfiguration | Medium | Default weights provided; admin changes logged in audit trail; simple/weighted average comparison flags large divergences |
| Over-reliance on similarity score in Stage 2 weighting | Low | Simple average computed alongside weighted for comparison; weights are tunable |